# Absolute and Relative Coordinate Practice

Draw a vertical and horizontal line through the center point of the graph paper. Label the X axis and the Y axis.

You will now simulate drawing lines in CAD using the absolute coordinate system. Draw the lines that would appear if the following commands were issued.

Problem #1					Problem #2

line 						line

from point 3,5					from point –15,5

to point 8,5					to point –5,15

to point 8,6					to point –15,15

to point 4,6					to point –5,5

to point 4,12					to point close

to point 8,12

to point 8,13

to point 3,13

to point close

Draw the following lines using relative coordinates. The starting point will be an absolute coordinate but all other points will be relative coordinates.

Problem #3					Problem #4

line 						line

from point 4, -3				from point –12, -4

to point @ 1, 0				to point  @ 4, -4

to point @ 1, -1				to point  @ 0, 1

to point @ 3, 0				to point  @ 4, -6

to point @ 1, 1				to point  @ -4, 4

to point @ 1, 0				to point  @ 0,-1

to point @ 0, -2				to point  close

to point @ -1, -1

to point @ 0, -6

to point @ -1,-1

to point @ 0, -3

to point @ -1, -1				Problem #5

to point @  0, -1				Make your own drawing on the back of the graph paper.

to point @  -1, 0				List the commands (using absolute or relative coord.)

to point @  0, 1				you would use to draw your object here.

to point @  -1, 1

to point @  0, 3

to point @  -1, 1

to point @  0, 6

to point @  -1, 1

to point close